# Trigger Board Assembly

System documentation: pin-by-pin reference, block diagram, board interconnect and bill of materials

Prepared 2026-10-06 from the released KiCad projects (Base_Board rev "pin version", Logic_Board rev 0.2). No design changes are implied by this document.

# 1. System overview

The Trigger Board Assembly is two stacked 1.6 mm boards: the Base board has 6 copper layers (F.Cu, In1-In4, B.Cu) and the Logic board has 4 (F.Cu, In1, In2, B.Cu). The Logic board (150 x 39 mm) plugs onto the Base board through two 2x9 connectors: Base J8 (socket) mates with Logic J1 (pin header), and Base J2 (socket) mates with Logic J2 (pin header). Each board has its own 24 V to 5 V DC-DC converter.

Functional summary, derived from the schematic netlist (verify against design intent):

Trigger path: an external trigger enters on BNC J6 or fiber header J10 (JP2 selects), optionally terminated in 50 ohm (JP3), and goes to the Logic board as Trig_In on J8 pin 1. The output pulse is driven by IC1 (IXDN614PI) as Trig_Out onto BNC J5 or fiber header J7 (JP1 selects; JP4 adds a 50 ohm load).

Gating: three opto-isolated external inhibit inputs (J1 pins 7, 9, 11) and the Logic board decision signal In_from_lgboard are ANDed in U6 to form the IC1 input. JP5 and JP6 can force two of the inhibit channels to 5 V.

Timing: U4 (74LS221) generates Master_TO_Out and Slave_TO_Out. Unit 1 uses C12 100 uF with R14 22 k (about 1.5 s, estimated 0.7 x R x C); unit 2 uses C4 1 nF with R13 1.5 k (about 1 us). U9 (74AHCT123) stretches the output pulse into Trig_Sts using C13 10 uF and R31 100 k (about 0.45 s, estimated 0.45 x R x C).

Status: Remote, Local, Armed, OneShot, Normal, Power, Trigger and inhibit-summary states are passed through quad optocouplers (U2, U7, U8) to the isolated terminal block J3, powered from the external 24 V (EXT_24V).

Mode selection: base switches SW2 and SW3 and push button SW1 drive the switch1/switch2 lines, the Arm/one-shot timer input and the Remote status net toward the Logic board.

Logic board: NAND/AND/OR logic (U2-U5 SN74ACT00, U6 74AHCT04, U7 74LS08, U8 4075) forms the armed / one-shot / normal / remote-local state logic and returns In_from_lgboard and OR_Out.

Figure 1. System block diagram (base + logic). Source: block_diagram.png in this folder.

# 2. Board-to-board interconnect

Base J8 plugs into Logic J1 and Base J2 plugs into Logic J2 (confirmed by the designer). Pins mate n to n; net names match on every populated pin of J8/J1, which supports this.

## 2.1 Base J8 to Logic J1

| Pin | Base J8 net | Logic J1 net | Note |
| --- | --- | --- | --- |
| 1 | Trig_In | Trig_In |  |
| 2 | GND | GND |  |
| 3 | OS_Select_EXT_Out | OS_Rem_Select_Out | name differs |
| 4 | In_from_lgboard | In_from_lgboard |  |
| 5 | Master_TO_Out | Master_TO |  |
| 6 | GND | GND |  |
| 7 | GND | GND |  |
| 8 | Slave_TO_Out | Slave_TO |  |
| 9 | NC | NC | spare |
| 10 | NC | NC | spare |
| 11 | NC | NC | spare |
| 12 | NC | NC | spare |
| 13 | NC | NC | spare |
| 14 | NC | NC | spare |
| 15 | NC | NC | spare |
| 16 | OR-In1 | OR-In1 |  |
| 17 | OR-In2 | OR-In2 |  |
| 18 | OR_Out | OR_Out |  |

## 2.2 Base J2 to Logic J2

| Pin | Base J2 net | Logic J2 net | Note |
| --- | --- | --- | --- |
| 1 | PWR_24V | 24V |  |
| 2 | GND | GND |  |
| 3 | GND | GND |  |
| 4 | NC | NC | spare |
| 5 | lg_5V_sts | 5V | name differs |
| 6 | switch2pin | Remote Sts | name differs |
| 7 | switch1pin | switch1pin |  |
| 8 | switch1opt | switch1opt |  |
| 9 | NC | NC | spare |
| 10 | NC | NC | spare |
| 11 | NC | NC | spare |
| 12 | NC | NC | spare |
| 13 | NC | NC | spare |
| 14 | Normal_Sts | Normal_Sts |  |
| 15 | OneShot_Sts | OneShot_Sts |  |
| 16 | Armed_Sts | Armed_Sts |  |
| 17 | Local_Sts | Local_Sts |  |
| 18 | Remote Sts | Remote Sts |  |

Notes on differing names (J2): pin 1 is the 24 V rail named PWR_24V on the base and 24V on the logic board. Pin 5 is the logic board 5 V rail, read on the base as lg_5V_sts (5 V present sense). Pin 6 is switch2pin on the base (SW2 output) and lands on the Logic Remote Sts net, which is also on pin 18; so SW2 on the base (the remote/local selector) drives the Logic board Remote Sts net, as intended (confirmed by the designer). Naming differences on J8 (Master_TO_Out vs Master_TO, Slave_TO_Out vs Slave_TO, OS_Select_EXT_Out vs OS_Rem_Select_Out, GND vs /GND) are the same signals.

# 3. Base board pinouts

## 3.1 J1: Field terminal block (Phoenix Contact 1700812, 18 positions): 24 V power, external isolated 24 V and inputs

| Pin | Function / pad name | Net |
| --- | --- | --- |
| 1 | 1 | PWR_24V |
| 2 | 2 | GND |
| 3 | 3 | EXT_24V |
| 4 | 4 | EXT_GND |
| 5 | 5 | OS_Select_EXT |
| 6 | 6 | Arm_OS_EXT |
| 7 | 7 | Inhibit1_EXT |
| 8 | 8 | NC |
| 9 | 9 | Inhibit2_EXT |
| 10 | 10 | Inhibit2_EXT_GND |
| 11 | 11 | Inhibit3_EXT |
| 12 | 12 | Inhibit3_EXT_GND |
| 13 | 13 | NC |
| 14 | 14 | NC |
| 15 | 15 | NC |
| 16 | 16 | NC |
| 17 | 17 | NC |
| 18 | 18 | NC |

## 3.2 J3: Field terminal block (Phoenix Contact 1700812, 18 positions): isolated status outputs

| Pin | Function / pad name | Net |
| --- | --- | --- |
| 1 | 1 | NC |
| 2 | 2 | NC |
| 3 | 3 | Rmt_Sts_FB |
| 4 | 4 | Local_Sts_FB |
| 5 | 5 | Inhib_Summ_FB |
| 6 | 6 | Armed_Sts_FB |
| 7 | 7 | Normal_Sts_FB |
| 8 | 8 | OneShot_Sts_FB |
| 9 | 9 | NC |
| 10 | 10 | Pwr_Sts_FB |
| 11 | 11 | NC |
| 12 | 12 | Trg_Sts_FB |
| 13 | 13 | NC |
| 14 | 14 | Inhibit1_Sts_FB |
| 15 | 15 | Inhibit2_Sts |
| 16 | 16 | Inhibit2_Sts_FB |
| 17 | 17 | Inhibit3_Sts |
| 18 | 18 | Inhibit3_Sts_FB |

## 3.3 J2: 2x9 socket to Logic J2

| Pin | Function / pad name | Net |
| --- | --- | --- |
| 1 | Pin_1 | PWR_24V |
| 2 | Pin_2 | GND |
| 3 | Pin_3 | GND |
| 4 | Pin_4 | NC |
| 5 | Pin_5 | lg_5V_sts |
| 6 | Pin_6 | switch2pin |
| 7 | Pin_7 | switch1pin |
| 8 | Pin_8 | switch1opt |
| 9 | Pin_9 | NC |
| 10 | Pin_10 | NC |
| 11 | Pin_11 | NC |
| 12 | Pin_12 | NC |
| 13 | Pin_13 | NC |
| 14 | Pin_14 | Normal_Sts |
| 15 | Pin_15 | OneShot_Sts |
| 16 | Pin_16 | Armed_Sts |
| 17 | Pin_17 | Local_Sts |
| 18 | Pin_18 | Remote Sts |

## 3.4 J8: 2x9 socket to Logic J1

| Pin | Function / pad name | Net |
| --- | --- | --- |
| 1 | Pin_1 | Trig_In |
| 2 | Pin_2 | GND |
| 3 | Pin_3 | OS_Select_EXT_Out |
| 4 | Pin_4 | In_from_lgboard |
| 5 | Pin_5 | Master_TO_Out |
| 6 | Pin_6 | GND |
| 7 | Pin_7 | GND |
| 8 | Pin_8 | Slave_TO_Out |
| 9 | Pin_9 | NC |
| 10 | Pin_10 | NC |
| 11 | Pin_11 | NC |
| 12 | Pin_12 | NC |
| 13 | Pin_13 | NC |
| 14 | Pin_14 | NC |
| 15 | Pin_15 | NC |
| 16 | Pin_16 | OR-In1 |
| 17 | Pin_17 | OR-In2 |
| 18 | Pin_18 | OR_Out |

## 3.5 J5: BNC, trigger output

| Pin | Function / pad name | Net |
| --- | --- | --- |
| 1 | In | (J5-In) |
| 2 | Ext | GND |

## 3.6 J6: BNC, trigger input

| Pin | Function / pad name | Net |
| --- | --- | --- |
| 1 | In | (J6-In) |
| 2 | Ext | GND |

## 3.7 J7: 3-pin header, fiber transmitter (output side) connection

| Pin | Function / pad name | Net |
| --- | --- | --- |
| 1 | Pin_1 | (J7-Pin_1) |
| 2 | Pin_2 | GND |
| 3 | Pin_3 | 5V |

## 3.8 J10: 3-pin header, fiber receiver (input side) connection

| Pin | Function / pad name | Net |
| --- | --- | --- |
| 1 | Pin_1 | (J10-Pin_1) |
| 2 | Pin_2 | GND |
| 3 | Pin_3 | 5V |

J7/J10 fiber header assignments are inferred from jumper wiring (JP1 B to J7 pin 1; JP2 B to J10 pin 1): J7 carries the output trigger, J10 the input trigger. Pin 1 = signal, pin 2 = GND, pin 3 = 5 V.

## 3.9 Jumpers and switches

| Ref | Pins and nets |
| --- | --- |
| JP1 | 1(A)=(J5-In); 2(C)=Trig_Out; 3(B)=(J7-Pin_1) |
| JP2 | 1(A)=(J6-In); 2(C)=Trig_In; 3(B)=(J10-Pin_1) |
| JP3 | 1(A)=(J6-In); 2(B)=(JP3-B) |
| JP4 | 1(A)=(JP4-A); 2(B)=(J5-In) |
| JP5 | 1(A)=5V; 2(B)=(JP5-B) |
| JP6 | 1(A)=5V; 2(B)=(JP6-B) |
| SW1 | 1(1)=(RN5A-R1.1); 2(2)=(U7-CT1) |
| SW2 | 1(B)=switch2pin; 2(C)=GND; 3(A)=5V |
| SW3 | 1(B)=switch1pin; 2(C)=GND; 3(A)=switch1opt |

# 4. Logic board pinouts

## 4.1 J1: 2x9 pin header to Base J8

| Pin | Net |
| --- | --- |
| 1 | Trig_In |
| 2 | GND |
| 3 | OS_Rem_Select_Out |
| 4 | In_from_lgboard |
| 5 | Master_TO |
| 6 | GND |
| 7 | GND |
| 8 | Slave_TO |
| 9 | NC |
| 10 | NC |
| 11 | NC |
| 12 | NC |
| 13 | NC |
| 14 | NC |
| 15 | NC |
| 16 | OR-In1 |
| 17 | OR-In2 |
| 18 | OR_Out |

## 4.2 J2: 2x9 pin header to Base J2

| Pin | Net |
| --- | --- |
| 1 | 24V |
| 2 | GND |
| 3 | GND |
| 4 | NC |
| 5 | 5V |
| 6 | Remote Sts |
| 7 | switch1pin |
| 8 | switch1opt |
| 9 | NC |
| 10 | NC |
| 11 | NC |
| 12 | NC |
| 13 | NC |
| 14 | Normal_Sts |
| 15 | OneShot_Sts |
| 16 | Armed_Sts |
| 17 | Local_Sts |
| 18 | Remote Sts |

# 5. Integrated circuits: pin-by-pin

Net names in parentheses are local nets named after a pin they connect to. NC = no connection.

### Base IC1: IXDN614PI trigger driver (OUT1/OUT2 paralleled)

| Pin | Function | Net |
| --- | --- | --- |
| 1 | VCC_1 | 5V |
| 2 | IN | (IC1-IN) |
| 3 | NC | NC |
| 4 | GND_1 | GND |
| 5 | GND_2 | GND |
| 6 | OUT_1 | Trig_Out |
| 7 | OUT_2 | Trig_Out |
| 8 | VCC_2 | 5V |

### Base U1: WPME-FDSM DC-DC, 24 V to 5 V

| Pin | Function | Net |
| --- | --- | --- |
| 1 | 1 | PWR_24V |
| 2 | 2 | GND |
| 3 | 3 | 5V |

### Base U2: PS2501L-4 optocoupler: status outputs

| Pin | Function | Net |
| --- | --- | --- |
| 1 | A1 | (RN3D-R4.1) |
| 2 | C1 | GND |
| 3 | A2 | (RN3E-R5.1) |
| 4 | C2 | GND |
| 5 | A3 | (RN4A-R1.1) |
| 6 | C3 | GND |
| 7 | A4 | (RN4B-R2.1) |
| 8 | C4 | GND |
| 9 | ET4 | OneShot_Sts_FB |
| 10 | CT4 | (RN5B-R2.2) |
| 11 | ET3 | Armed_Sts_FB |
| 12 | CT3 | (RN2F-R6.2) |
| 13 | ET2 | Local_Sts_FB |
| 14 | CT2 | (RN2E-R5.2) |
| 15 | ET1 | Rmt_Sts_FB |
| 16 | CT1 | (RN2D-R4.2) |

### Base U3: PS2501L-4 optocoupler: external inputs (OS select, inhibits)

| Pin | Function | Net |
| --- | --- | --- |
| 1 | A1 | (RN1A-R1.2) |
| 2 | C1 | EXT_GND |
| 3 | A2 | (RN1B-R2.2) |
| 4 | C2 | EXT_GND |
| 5 | A3 | (RN1C-R3.2) |
| 6 | C3 | Inhibit2_EXT_GND |
| 7 | A4 | (RN1D-R4.2) |
| 8 | C4 | Inhibit3_EXT_GND |
| 9 | ET4 | (JP5-B) |
| 10 | CT4 | 5V |
| 11 | ET3 | (JP6-B) |
| 12 | CT3 | 5V |
| 13 | ET2 | (RN6D-R4.2) |
| 14 | CT2 | 5V |
| 15 | ET1 | GND |
| 16 | CT1 | OS_Select_EXT_Out |

### Base U4: 74LS221 dual monostable

| Pin | Function | Net |
| --- | --- | --- |
| 1 | ~{A} | GND |
| 2 | B | (RN5A-R1.1) |
| 3 | ~{CLR} | 5V |
| 4 | ~{Q} | NC |
| 5 | Q | NC |
| 6 | Cext | (U4B-Cext) |
| 7 | RCext | (U4B-RCext) |
| 8 | GND | GND |
| 9 | ~{A} | GND |
| 10 | B | Master_TO_Out |
| 11 | ~{CLR} | 5V |
| 12 | ~{Q} | Slave_TO_Out |
| 13 | Q | Master_TO_Out |
| 14 | Cext | (U4A-Cext) |
| 15 | RCext | (U4A-RCext) |
| 16 | VCC | 5V |

### Base U5: 74AHCT04 hex inverter

| Pin | Function | Net |
| --- | --- | --- |
| 1 |  | (RN6D-R4.2) |
| 2 |  | (RN3A-R1.2) |
| 3 |  | (JP6-B) |
| 4 |  | (RN3B-R2.2) |
| 5 |  | (JP5-B) |
| 6 |  | (RN3C-R3.2) |
| 7 | GND | GND |
| 8 |  | OR-In2 |
| 9 |  | lg_5V_sts |
| 10 |  | OR-In1 |
| 11 |  | 5V |
| 12 |  | Pwr_Sts |
| 13 |  | OR_Out |
| 14 | VCC | 5V |

### Base U6: 74LS08 quad AND

| Pin | Function | Net |
| --- | --- | --- |
| 1 |  | (RN6D-R4.2) |
| 2 |  | (JP6-B) |
| 3 |  | (RN8B-R2.2) |
| 4 |  | (RN8B-R2.2) |
| 5 |  | (JP5-B) |
| 6 |  | (RN8C-R3.2) |
| 7 | GND | GND |
| 8 |  | Inhib_Sum |
| 9 |  | (RN8C-R3.2) |
| 10 |  | 5V |
| 11 |  | (IC1-IN) |
| 12 |  | In_from_lgboard |
| 13 |  | (RN8C-R3.2) |
| 14 | VCC | 5V |

### Base U7: PS2501L-4 optocoupler: Arm input and inhibit status

| Pin | Function | Net |
| --- | --- | --- |
| 1 | A1 | (RN1E-R5.2) |
| 2 | C1 | EXT_GND |
| 3 | A2 | (RN3A-R1.1) |
| 4 | C2 | GND |
| 5 | A3 | (RN3B-R2.1) |
| 6 | C3 | GND |
| 7 | A4 | (RN3C-R3.1) |
| 8 | C4 | GND |
| 9 | ET4 | Inhibit3_Sts_FB |
| 10 | CT4 | (RN2C-R3.2) |
| 11 | ET3 | Inhibit2_Sts_FB |
| 12 | CT3 | (RN2B-R2.2) |
| 13 | ET2 | Inhibit1_Sts_FB |
| 14 | CT2 | (RN2A-R1.2) |
| 15 | ET1 | (RN5A-R1.1) |
| 16 | CT1 | (U7-CT1) |

### Base U8: PS2501L-4 optocoupler: status outputs

| Pin | Function | Net |
| --- | --- | --- |
| 1 | A1 | (RN4C-R3.1) |
| 2 | C1 | GND |
| 3 | A2 | (RN4D-R4.1) |
| 4 | C2 | GND |
| 5 | A3 | (RN4E-R5.1) |
| 6 | C3 | GND |
| 7 | A4 | (RN4F-R6.1) |
| 8 | C4 | GND |
| 9 | ET4 | Inhib_Summ_FB |
| 10 | CT4 | (RN5F-R6.2) |
| 11 | ET3 | Trg_Sts_FB |
| 12 | CT3 | (RN5E-R5.2) |
| 13 | ET2 | Pwr_Sts_FB |
| 14 | CT2 | (RN5D-R4.2) |
| 15 | ET1 | Normal_Sts_FB |
| 16 | CT1 | (RN5C-R3.2) |

### Base U9: 74AHCT123 dual monostable

| Pin | Function | Net |
| --- | --- | --- |
| 1 | A | GND |
| 2 | B | Trig_Out |
| 3 | Clr | 5V |
| 4 | ~{Q} | NC |
| 5 | Q | NC |
| 6 | Cext | NC |
| 7 | RCext | NC |
| 8 | GND | GND |
| 9 | A | NC |
| 10 | B | NC |
| 11 | Clr | NC |
| 12 | ~{Q} | NC |
| 13 | Q | Trig_Sts |
| 14 | Cext | GND |
| 15 | RCext | (U9A-RCext) |
| 16 | VCC | 5V |

### Logic U1: WPME-FDSM DC-DC, 24 V to 5 V

| Pin | Function | Net |
| --- | --- | --- |
| 1 | 1 | 24V |
| 2 | 2 | GND |
| 3 | 3 | 5V |

### Logic U2: SN74ACT00

| Pin | Function | Net |
| --- | --- | --- |
| 1 |  | (U2-Pad1) |
| 2 |  | Master_TO |
| 3 |  | (U2-Pad3) |
| 4 |  | (U2-Pad3) |
| 5 |  | 5V |
| 6 |  | (U2-Pad13) |
| 7 | GND | GND |
| 8 |  | (U2-Pad12) |
| 9 |  | Slave_TO |
| 10 |  | (U2-Pad10) |
| 11 |  | (U2-Pad10) |
| 12 |  | (U2-Pad12) |
| 13 |  | (U2-Pad13) |
| 14 | VCC | 5V |

### Logic U3: SN74ACT00

| Pin | Function | Net |
| --- | --- | --- |
| 1 |  | (U2-Pad12) |
| 2 |  | Trig_In |
| 3 |  | (U3-Pad3) |
| 4 |  | (U3-Pad4) |
| 5 |  | Trig_In |
| 6 |  | (U3-Pad12) |
| 7 | GND | GND |
| 8 |  | (U3-Pad4) |
| 9 |  | (U2-Pad12) |
| 10 |  | 5V |
| 11 |  | (U3-Pad11) |
| 12 |  | (U3-Pad12) |
| 13 |  | Master_TO |
| 14 | VCC | 5V |

### Logic U4: SN74ACT00

| Pin | Function | Net |
| --- | --- | --- |
| 1 |  | (U3-Pad3) |
| 2 |  | (U4-Pad2) |
| 3 |  | (U4-Pad3) |
| 4 |  | (U4-Pad3) |
| 5 |  | (U4-Pad5) |
| 6 |  | (U4-Pad2) |
| 7 | GND | GND |
| 8 |  | (U4-Pad5) |
| 9 |  | (U3-Pad11) |
| 10 |  | 5V |
| 11 |  | (U4-Pad11) |
| 12 |  | Trig_In |
| 13 |  | 5V |
| 14 | VCC | 5V |

### Logic U5: SN74ACT00

| Pin | Function | Net |
| --- | --- | --- |
| 1 |  | (U4-Pad3) |
| 2 |  | (U4-Pad11) |
| 3 |  | (U5-Pad3) |
| 4 |  | (U4-Pad2) |
| 5 |  | (U4-Pad11) |
| 6 |  | (U5-Pad13) |
| 7 | GND | GND |
| 8 |  | Armed_Sts |
| 9 |  | (U5-Pad3) |
| 10 |  | (U2-Pad1) |
| 11 |  | (U2-Pad1) |
| 12 |  | Armed_Sts |
| 13 |  | (U5-Pad13) |
| 14 | VCC | 5V |

### Logic U6: 74AHCT04

| Pin | Function | Net |
| --- | --- | --- |
| 1 |  | Remote Sts |
| 2 |  | Local_Sts |
| 3 |  | OneShot_Sts |
| 4 |  | Normal_Sts |
| 5 |  | OS_Rem_Select_Out |
| 6 |  | (U6-Pad6) |
| 7 | GND | GND |
| 8 |  | (U6-Pad8) |
| 9 |  | switch1pin |
| 10 |  | (U6-Pad10) |
| 11 |  | Remote Sts |
| 12 |  | switch1opt |
| 13 |  | Remote Sts |
| 14 | VCC | 5V |

### Logic U7: 74LS08

| Pin | Function | Net |
| --- | --- | --- |
| 1 |  | Remote Sts |
| 2 |  | (U6-Pad6) |
| 3 |  | (U7-Pad3) |
| 4 |  | (U6-Pad6) |
| 5 |  | (U6-Pad8) |
| 6 |  | (U7-Pad6) |
| 7 | GND | GND |
| 8 |  | (U7-Pad8) |
| 9 |  | (U6-Pad8) |
| 10 |  | (U6-Pad10) |
| 11 |  | In_from_lgboard |
| 12 |  | Trig_In |
| 13 |  | (U7-Pad13) |
| 14 | VCC | 5V |

### Logic U8: 4075 triple OR

| Pin | Function | Net |
| --- | --- | --- |
| 1 |  | (U7-Pad3) |
| 2 |  | (U7-Pad6) |
| 3 |  | switch1pin |
| 4 |  | Armed_Sts |
| 5 |  | OS_Rem_Select_Out |
| 6 |  | (U7-Pad13) |
| 7 | VSS | GND |
| 8 |  | (U7-Pad8) |
| 9 |  | OneShot_Sts |
| 10 |  | OR_Out |
| 11 |  | OR-In1 |
| 12 |  | OR-In2 |
| 13 |  | GND |
| 14 | VDD | 5V |

For 74xx devices, the gate-level pin functions follow the standard pinouts; the table lists the nets as routed.

# 6. Passive networks

| Board | Ref | Value | Pin nets |
| --- | --- | --- | --- |
| Base | C1 | C_Polarized_US | 1=PWR_24V; 2=GND |
| Base | C10 | 100n | 1=5V; 2=GND |
| Base | C11 | 100n | 1=GND; 2=5V |
| Base | C12 | 100u | 1=(U4A-Cext); 2=(U4A-RCext) |
| Base | C13 | 10u | 1=(U9A-RCext); 2=GND |
| Base | C2 | C_Polarized_US | 1=5V; 2=GND |
| Base | C3 | 100n | 1=GND; 2=5V |
| Base | C4 | 1n | 1=(U4B-Cext); 2=(U4B-RCext) |
| Base | C5 | 100n | 1=(RN5A-R1.1); 2=GND |
| Base | C6 | 100n | 1=GND; 2=5V |
| Base | C7 | 100n | 1=GND; 2=5V |
| Base | R13 | 1k5 | 1=(U4B-RCext); 2=5V |
| Base | R14 | 22k | 1=(U4A-RCext); 2=5V |
| Base | R16 | 100 | 1=(U7-CT1); 2=5V |
| Base | R30 | 50_1W | 1=(JP4-A); 2=GND |
| Base | R31 | 100k | 1=(U9A-RCext); 2=5V |
| Base | R32 | 50_1W | 1=(JP3-B); 2=GND |
| Base | RN1 | 4k7 | 1=OS_Select_EXT; 2=(RN1A-R1.2); 3=Inhibit1_EXT; 4=(RN1B-R2.2); 5=Inhibit2_EXT; 6=(RN1C-R3.2); 7=Inhibit3_EXT; 8=(RN1D-R4.2); 9=Arm_OS_EXT; 10=(RN1E-R5.2) |
| Base | RN2 | 1k | 1=EXT_24V; 2=(RN2A-R1.2); 3=Inhibit2_Sts; 4=(RN2B-R2.2); 5=Inhibit3_Sts; 6=(RN2C-R3.2); 7=EXT_24V; 8=(RN2D-R4.2); 9=EXT_24V; 10=(RN2E-R5.2); 11=EXT_24V; 12=(RN2F-R6.2) |
| Base | RN3 | 470 | 1=(RN3A-R1.1); 2=(RN3A-R1.2); 3=(RN3B-R2.1); 4=(RN3B-R2.2); 5=(RN3C-R3.1); 6=(RN3C-R3.2); 7=(RN3D-R4.1); 8=Remote Sts; 9=(RN3E-R5.1); 10=Local_Sts |
| Base | RN4 | 470 | 1=(RN4A-R1.1); 2=Armed_Sts; 3=(RN4B-R2.1); 4=OneShot_Sts; 5=(RN4C-R3.1); 6=Normal_Sts; 7=(RN4D-R4.1); 8=Pwr_Sts; 9=(RN4E-R5.1); 10=Trig_Sts; 11=(RN4F-R6.1); 12=Inhib_Sum |
| Base | RN5 | 1k | 1=(RN5A-R1.1); 2=GND; 3=EXT_24V; 4=(RN5B-R2.2); 5=EXT_24V; 6=(RN5C-R3.2); 7=EXT_24V; 8=(RN5D-R4.2); 9=EXT_24V; 10=(RN5E-R5.2); 11=EXT_24V; 12=(RN5F-R6.2) |
| Base | RN6 | 10k | 1=GND; 2=NC; 3=OR_Out; 4=(JP6-B); 5=(RN6D-R4.2) |
| Base | RN7 | 10k | 1=GND; 2=(RN6D-R4.2); 3=(JP6-B); 4=(JP5-B); 5=lg_5V_sts |
| Base | RN8 | 10k | 1=GND; 2=(JP5-B); 3=(RN8B-R2.2); 4=(RN8C-R3.2); 5=In_from_lgboard |
| Logic | C1 | C_Polarized_US | 1=5V; 2=GND |
| Logic | C2 | C_Polarized_US | 1=24V; 2=GND |
| Logic | C3 | 100n | 1=GND; 2=5V |
| Logic | C4 | 100n | 1=GND; 2=5V |
| Logic | C5 | 100n | 1=GND; 2=5V |
| Logic | C6 | 100n | 1=GND; 2=5V |
| Logic | C7 | 100n | 1=GND; 2=5V |
| Logic | C8 | 100n | 1=GND; 2=5V |
| Logic | C9 | 100n | 1=GND; 2=5V |
| Logic | R1 | 10k | 1=Remote Sts; 2=OS_Rem_Select_Out |

# 7. Bill of materials

Full BOM, with base and logic breakouts, is in Trigger_Board_System_BOM.xlsx. System totals below (R: ref prefix B = Base, L = Logic).

| Qty | Ref(s) | Schematic value | Description | MFR part # | Mouser # |
| --- | --- | --- | --- | --- | --- |
| 2 | B:C1, B:C2 | C_Polarized_US | 100uF 50V Electrolythic Capacitor | ESL107M050AGMAA | 80-ESL107M050AGMAA |
| 6 | B:C3, B:C5, B:C6, B:C7, B:C10, B:C11 | 100n | 100nF Ceramic disk capacitor | S104Z93Z5VL83L0R | 594-S104Z93Z5VL83L0R |
| 1 | B:C4 | 1n | 1nF Ceramic Disk Capacitor | FA28C0G1H102JNU00 | 810-FA28C0G1H10200 |
| 1 | B:C12 | 100u | 100uF 25V Electrolythic Capacitor | Kit part (no part number) | NA |
| 1 | B:C13 | 10u | 10uF Ceramic Disk Capacitor | FG11X7R1H106KRT00 | 810-FG11X7R1H106KRT0 |
| 1 | B:IC1 | IXDN614PI | IXYS MOSFET Gate Driver | IXDN614PI | 849-IXDN614PI |
| 1 | B:J1 | J6 | Screw Terminal Blocks | 1725041 | 651-1725041 |
| 2 | B:J2, B:J8 | Conn_02x09_Counter_Clockwise | 02x09 Pin Socket | 215307-9 | 571-215307-9 |
| 1 | B:J3 | 1700812 | Screw Terminal Blocks | 1725041 | 651-1725041 |
| 2 | B:J5, B:J6 | Conn_Coaxial | BNC female PCB mounted Connector | 1-1478204-0 | 571-1-1478204-0 |
| 2 | B:J7, B:J10 | Conn_01x03_Pin | Fiber RX/TX Eval boards. | AFBR-0548Z | 630-AFBR-0548Z |
| 2 | B:JP1, B:JP2 | Jumper_3_Open | Jumper Pins (2 pin ones must be modded) - 6 used + 1 spare | 90120-0763 | 538-90120-0763 |
| 4 | B:JP3, B:JP4, B:JP5, B:JP6 | Jumper_2_Small_Open | Jumper Pins (2 pin ones must be modded) - 6 used + 1 spare | 90120-0763 | 538-90120-0763 |
| 1 | B:R13 | 1k5 | 1K5 ohm 1/4W | MFR-25FBF52-1K5 | 603-MFR-25FBF52-1K5 |
| 1 | B:R14 | 22k | 22Kohm 1/4W | MFR-25FTF52-22K | 603-MFR-25FTF52-22K |
| 1 | B:R16 | 100 | 100 ohm 1/4W | MFR-25FBF52-100R | 603-MFR-25FBF52-100R |
| 2 | B:R30, B:R32 | 50_1W | 50-Ohm 1W Resistor | PAC100005009FA1000 | 71-PAC100005009FA100 |
| 1 | B:R31 | 100k | 100K ohm 1/4W | MFR-25FTE52-100K | 603-MFR-25FTE52-100K |
| 1 | B:RN1 | 4k7 | Resistor Network Isolated(4k7x05) | 770103472P | 774-770103472P |
| 2 | B:RN2, B:RN5 | 1k | Resistor Network Isolated(1k06) | 4612X-102-102LF | 652-4612X-2LF-1K |
| 1 | B:RN3 | 470 | Resistor Network Isolated (470x05) | 4610X-102-471LF | 652-4610X-2LF-470 |
| 1 | B:RN4 | 470 | Resistor Network Isolated (470x06) | 4612X-102-471LF | 652-4612X-2LF-470 |
| 3 | B:RN6, B:RN7, B:RN8 | 10k | Resistor Network Bussed(10Kx04) | 4605X-101-103FLF | 652-4605X-101-103FLF |
| 1 | B:SW1 | SW_Push | Tactile switch | TS02-66-60-BK-260-LCR-D | 179-TS026660BK260LCR |
| 2 | B:SW2, B:SW3 | SW_Wuerth_450301014042 | Wuerth Switch | 450301014042 | 710-450301014042 |
| 2 | B:U1, L:U1 | WPME-FDSM_SIP-3 | Wuerth Electronik Voltage Regulator | 173010578 | 710-173010578 |
| 4 | B:U2, B:U3, B:U7, B:U8 | PS2501L-4-A | 4-Channel OptoCoupler | PS2501-4-A | 551-PS2501-4-A |
| 1 | B:U4 | 74LS221 | Texas Instruments Monostable Multivibrator | SN74LS221NE4 | 595-SN74LS221NE4 |
| 2 | B:U5, L:U6 | 74AHCT04 | Texas Instruments Hex Inverter | SN74AHCT04N | 595-SN74AHCT04N |
| 1 | B:U6 | 74LS08 | Texas Instruments 4-Gate AND | SN74LS08N | 595-SN74LS08N |
| 1 | B:U9 | 74AHCT123 | Texas Instruments Monostable Multivibrator | SN74AHCT123AN | 595-SN74AHCT123AN |
| 2 | L:C1, L:C2 | C_Polarized_US | 100uF 50V Electrolythic Capacitor | ESL107M050AGMAA | 80-ESL107M050AGMAA |
| 7 | L:C3, L:C4, L:C5, L:C6, L:C7, L:C8, L:C9 | 100n | 100 nF Ceramic disk capacitor | S104Z93Z5VL83L0R | 594-S104Z93Z5VL83L0R |
| 2 | L:J1, L:J2 | Conn_02x09_Counter_Clockwise | 02x09 Pin Header | 826925-9 | 571-826925-9 |
| 1 | L:R1 | 10k | 10K 1% 1/4W Resistor | Kit part (no part number) | NA |
| 4 | L:U2, L:U3, L:U4, L:U5 | SN74ACT00 | Texas Instruments Quad 2-Input NAND | SN74ACT00N | 595-SN74ACT00N |
| 1 | L:U7 | 74LS08 | Texas Instruments 4-Gate AND | SN74LS08N | 595-SN74LS08N |
| 1 | L:U8 | 4075 | Texas Instruments Triple 3-input OR Gate | CD4075BE | 595-CD4075BE |

Not on the PCBs (see the Off-Board Items sheet of the BOM spreadsheet): the AFBR-0548Z fiber RX/TX evaluation board (qty 1, plugs into J7 and J10, modified), Phoenix Contact housing parts (2278500, 2278526, 2278542, 2 x 2896238), 6 jumper shunts (JM-2BK-61) and the bare PCBs. The source of part numbers is Final_Trigger_Board_BOM.xlsx.

Total: 72 parts (52 base, 20 logic), excluding bare PCBs, shunts, sockets and hardware.

# 8. BOM reconciliation and open items

Part numbers above come from Final_Trigger_Board_BOM.xlsx matched by reference designator. Every schematic reference appears in that BOM. Differences found:

C4 and C13: the original Final BOM had the designators swapped. The board is correct (C4 marked 102 = 1 nF, C13 marked 106 = 10 uF, as in the schematic) and Final_Trigger_Board_BOM.xlsx is now corrected.

C12: the fitted part is a regular 100 uF 25 V electrolytic from a capacitor kit (no part number), measured D 5.18 mm, H 11.37 mm, lead pitch 1.77 mm as bent, with its leads bent so the top PCB does not hit it. This matches the schematic. The original Final BOM MPN reads as 2.2 uF / 100 V and was replaced with kit part.

Logic R1: the fitted resistor is 10 k, 1% (bands brown-black-black-red-brown), from a resistor kit with no part number. This matches the schematic. The original Final BOM MPN reads as 6.8 k and was replaced with kit part.

The original Final BOM listed JP7 and JP10. These are the 3-pin 2.54 mm positions where the eval board pins are soldered, i.e. J7 and J10 in the schematic, not extra jumpers; they are not jumpers. There are 6 jumpers (JP1-JP6); the Final BOM quantity of 7 is 6 used plus 1 spare, so item 10 reads JP1-JP6, quantity 7 (6 + 1 spare).

Logic U1 Mouser number was corrected to 710-173010578 to match base U1, and base U5 to 595-SN74AHCT04N (missing prefix).

One AFBR-0548Z evaluation board (qty 1) is listed for both J7 and J10. The bent interface pins were removed and replaced with straight pins facing vertically down (longer part of the pins on the side opposite the fiber ports); the replacement pins are generic 3-position, 2.54 mm pitch header (no part number) cut from a longer strip on hand.

Terminal blocks J1 and J3: the BOM lists 6 blocks of MPN 1725041 for 18 positions each, which resolves the earlier library-text/footprint question if that part is 3-position (not verified).

Schematic symbol U2/U3/U7/U8 is PS2501L-4-A; the ordered part is PS2501-4-A in a DIP-16 footprint.

Mating of J8/J1 and J2/J2 is confirmed by the designer (section 2). Functional descriptions in section 1 are derived from the netlist, not from design intent.

# 8.1 Assembly notes

Fiber evaluation board (AFBR-0548Z): the bent interface pins were removed and replaced with straight pins facing vertically down, the longer part of the pins on the side opposite the fiber ports.

Logic R1 (10 k, 1%): a resistor from a resistor kit (bands brown-black-black-red-brown), no part number.

C12 (100 uF, 25 V): a regular electrolytic capacitor from an Amazon capacitor kit (no part number; D 5.18 mm, H 11.37 mm, lead pitch 1.77 mm as bent), with its leads bent so the top (logic) board does not hit it.

# 8.2 Final testing (from the Final Testing sheet)

Two assemblies were tested against 22 items. All items passed on both assemblies, including BNC and fiber input/output, inhibit 1-3, the one-shot button, local/remote and normal/one-shot switches, and the 50-ohm termination jumpers.

| Test | Description | Assy 1 | Assy 2 |
| --- | --- | --- | --- |
| 50-ohm termination jumpers | 50 ohms across BNC connectors when jumper present. BNC open if jumper removed. | Pass | Pass |
| Local-Remote switch | Local and Remote status outputs toggle properly with switch input. | Pass | Pass |
| Normal-OS switch | Normal and OS feedbacks are triggered appropiately. | Pass | Pass |
| Power status feedback | HIGH(24V) when PWR and EXT PWR are present | Pass | Pass |
| Inhibbit Summation Status (inh2 jumper) | Triggered on inh2 jumper | Pass | Pass |
| Inhibbit Summation Status (inh3 jumper) | Triggered on inh3 jumper | Pass | Pass |
| Inhibit 1 feedback | triggers HIGH if inhibit 1 input LO and viceversa | Pass | Pass |
| input selection jumper | Propely labeled and selects the appropiate input | Pass | Pass |
| output selection jumper | Propely labeled and selects the appropiate output | Pass | Pass |
| Inhibit 2 jumper | Stops Pulses and relfects properly on INH summation | Pass | Pass |
| Inhibit 3 jumper | Stops Pulses and relfects properly on INH summation | Pass | Pass |
| Trigger_sts_fb | tested on BNC triggering | Pass | Pass |
| OS button | allows one pulse when proper switch setup and button pressede | Pass | Pass |
| inhibit 2 in/out | inhibit 2 input signal propelry closes the selected voltage on INH2 feedback. | Pass | Pass |
| inhibit 3 in/out | inhibit 3 input signal propelry closes the selected voltage on INH3 feedback. | Pass | Pass |
| Remote OS signal input | When swtiches are appropiate, remote OS is selected and Remote OS Signal is received a single pulse is observed at the output | Pass | Pass |
| BNC input | Input works properly | Pass | Pass |
| BNC output | Output works properly | Pass | Pass |
| Fiber input | Input works properly | Pass | Pass |
| fiber output | Output works properly | Pass | Pass |
| INH1 stops pulses | When pulsing, INH1 LOW stops the pulses. | Pass | Pass |
| INH2 stops pulses | When Pulsing INH2 LOW stops the pulses. | Pass | Pass |
| INH3 stops pulses | When Pulsing INH3 LOW stops the pulses. | Pass | Pass |

# 9. Gerber verification (released Gerbers vs KiCad project files)

Checked 2026-10-06 by parsing the released Gerber/drill zips (generated 2025-05-30 by KiCad 8.0.8) and comparing them with Base_Board/KiCad/baseboard.kicad_pcb and Logic_Board/KiCad/logicboard.kicad_pcb. KiCad itself was not available, so the check compares geometry; it is not a byte-for-byte regeneration.

| Item | Base board | Logic board |
| --- | --- | --- |
| Copper tracks, every layer | all 966 match exactly (position, width) | all 369 match exactly |
| Zone fills | 22 of 22 identical | 3 of 3 present (count match) |
| Drill holes (PTH) | 389 of 389 within 1 um; no NPTH holes in either file | 173 of 173 exact |
| Board outline | 148 lines + 48 arcs, count match | 4 lines, count match |
| Solder-mask openings | 352 flashes, all at pad positions | 157 flashes, all at pad positions |
| Not verified | silkscreen, paste, pad/aperture sizes and shapes | same |

Result: copper, drills, outline and mask openings in the released Gerbers match the current project files. The base PCB file was last saved 2025-06-17, after the 2025-05-30 Gerbers, but no geometry difference was found, so those later saves did not change the checked items.